ARPANET IMP and TIP Extensions

for the H316 simulator

December 2, 2013

COPYRIGHT NOTICE

NOTE

This work is based on the simh simulator package written by Robert M Supnik.  The copyright on those files is owned by Mr. Supnik and they are distributed under the terms of his license.  Please refer to the simh H316 emulator documentation or any of the associated source files for a complete copy of this license.  The following copyright notice applies only to the IMP/TIP extensions to the simh package.

Copyright © 2013 by Robert Armstrong, bob@jfcl.com.

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions: The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL ROBERT ARMSTRONG BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

Except as contained in this notice, the name of Robert Armstrong shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert Armstrong.

1	Introduction	3

2	Simulator Files	3

3	Additional Features and Devices	3

3.1.1	IMP Pseudo Device	3

3.1.2	Real Time Clock	4

3.1.3	Watch Dog Timer	5

3.2	Communications Devices	5

3.2.1	Modem Interface	5

3.2.2	Host Interface	7

4	Summary	9

4.1	Additional Devices	9

4.2	Modem Interface I/O Instructions	10

4.3	Host Interface  I/O Instructions	10

4.4	Other I/O Instructions	10

# Introduction

This memorandum documents the extensions made to the simh Honeywell H316 simulator to allow it to run the ARPANET Interface Message Processor (aka IMP) and Terminal Interface Processor (TIP) software.  A single IMP or TIP instance isn’t very useful, however these extensions allow multiple simh instances, each running a copy of the IMP or TIP code, to communicate using virtual modem connections implemented with physical serial ports on the host computer.  Alternatively, virtual modem connections may be tunneled over TCP/IP to a remote simh IMP/TIP instance anywhere in the world.   Moreover, each simh IMP instance can then be connected via virtual host interface cards to other local PDP-10 or PDP-11 simh instances running ARPANET host software.

NOTE

Presently TIP support is not implemented, and IMP host card support is only partially implemented.  The current implementation is sufficient to allow multiple IMPs to talk via virtual modem connections, and that’s all.

# Simulator Files

These additional IMP/TIP specific files are added to simh:

sim/h316/	h316_imp.h	IMP/TIP and ARPAnet specific definitions

h316_imp.c	the IMP pseudo device

h316_rtc.c	real time clock and watch dog timer

h316_hi.c	host interfaces

h316_mi.c	modem interfaces

h316_udp.c	UDP support for MI and HI modules

# Additional Features and Devices

The IMP/TIP adds the following devices to the H316 configuration:

device name		simulates

MI			modem interface (up to 5)

HI			host interface (up to 4)

WDT			watchdog timer

RTC			real time clock (replaces H316 CLK device)

IMP			TASK, IMPN and MLC functions (addresses 418 and 428)

### IMP Pseudo Device

The IMP pseudo device implements two sets of miscellaneous I/O instructions in the original IMP/TIP hardware.  This device responds both to IO address 418, which implements the TASK switching interrupt and the IMP identification number, and to IO address 428, which implements the MLC test.  The IMP device may be enabled with the standard command

SET IMP ENABLED

The IMP address, which is returned by the RDIMPN (INA 1041) instruction, may be set with the command

SET IMP NUM=n			set IMP station address to n

This value was apparently hardwired into each original IMP/TIP station.  The IMP device also implements the AMIMLC (SKS 0042) instruction, which skips if the current hardware is a multi-line controller (aka a TIP).  Since TIP emulation is not currently supported, this instruction is presently hard wired as a NOP (i.e. it never skips).

The IMP device implements the following registers:

name			size		comments

MLC			1		always zero (TIP flag)

IEN			1		task interrupt enabled

IRQ			1		task interrupt pending

These registers can be viewed with the command

EXAMINE IMP STATE

The IMP device implements these debugging flags:

SET IMP DEBUG=WARN		print warnings for unusual conditions

SET IMP DEBUG=IO			trace all IMP device I/O instructions

Remember that you must enable debugging output first before these settings will be effective; refer to the SIMH User’s Guide, “Controlling Debugging,” for more information.

### Real Time Clock

The IMP/TIP hardware contained a custom real time clock which was unique to that implementation, and further, the IMP/TIP did not have the standard H316 power line clock or real time clock. The IMP/TIP clock is in some ways similar to the H316 real time clock option, but it is not the same.  Normally the CLK device would be disabled when emulating the IMP, however there is no actual conflict between the CLK and the RTC devices and both can be enabled simultaneously.

The RTC device supports the following SET commands:

SET RTC ENABLED			enable RTC emulation

SET RTC INTERVAL=i		set the RTC tick interval to i microseconds

SET RTC QUANTUM=q			set the RTC resolution to q ticks

The INTERVAL parameter sets the interval between RTC clock ticks, in microseconds and the QUANTUM parameter sets how often, as a number of clock ticks, simh updates the RTC count.  To give an example, if INTERVAL was set to 20 and QUANTUM was 1, simh would attempt to increment the RTC count by one at a rate of 50,000 times per second.  Modern PCs are very fast and it would probably be possible for simh to do this, but the amount of overhead would be huge and it would be impractical.

Instead, for example, if INTERVAL was still set to 20 but QUANTUM was now set to 50, simh would add 50 to the RTC count at a rate of 1000 times per second.  This represents much less overhead and yet gives the same net counting rate as before.  However, now the clock count jumps in increments of 50 rather than 1.  Most software wouldn’t notice, but it’s always possible this could cause problems.

It’s important to note that the QUANTUM does not affect the overall clock frequency – as long as the INTERVAL is set to 20 the clock would count at an effective rate of 50 kHz regardless of the QUANTUM value.

The RTC device implements the following registers:

name			size		comments

ENA			 1		RTC is enabled

COUNT			16		current count

IEN			 1		RTC interrupt enabled

IRQ			 1		RTC interrupt pending

TPS			 1		effective ticks per second

WAIT			24		simulator time until the next tick

These registers can be viewed with the command

EXAMINE RTC STATE

The RTC device implements these debugging flags:

SET RTC DEBUG=WARN		print warnings for unusual conditions

SET RTC DEBUG=IO			trace all RTC device I/O instructions

Please refer to the SIMH User’s Guide, “Controlling Debugging,” for more information and remember that you must enable debugging output first before these settings will be effective.

### Watch Dog Timer

The IMP/TIP hardware also had a custom watch dog timer implementation which would force a non-maskable interrupt if was ever allowed to expire.  The WDT device in simh implements this feature.  The IMP/TIP watch dog timer also implemented a couple of other unrelated features – these include the status display panel, which you can see featured prominently in some IMP photos, and a flag to indicate whether the CPU was a H316 or DDP-516.

The WDT device supports the following SET commands

SET WDT ENABLED			enable WDT emulation

SET WDT DELAY=n			set the WDT delay to n milliseconds

The DELAY parameter sets the WDT timeout, in milliseconds.  This is a 16 bit unsigned value, so the maximum WDT delay is just over a minute.  Note that there are no H316 instructions that disable the WDT – only one to reset it – so if the WDT device is enabled the WDT will run anytime simulation is active.  If the code fails to reset the WDT in a timely fashion, the WDT interrupt will occur regardless of the enabled or disabled state of the interrupt system.

Setting the WDT delay to zero

SET WDT DELAY=0

prevents the WDT from ever generating a timeout.  This special feature allows the WDT device to be enabled, so that code which executes WDT, status light or AMI512 instructions can be executed and debugged, but without inadvertently triggering a WDT timeout.

The WDT device implements the following registers

name			size		comments

COUNT			16		current countdown

TMO			 1		WDT timed out

LIGHTS		16		last “set status lights”

WAIT			24		calcuated time until the next tick

These registers can be viewed with the command

EXAMINE WDT STATE

The WDT device implements these debugging flags:

SET WDT DEBUG=WARN		print warnings for unusual conditions

SET WDT DEBUG=IO			trace all WDT device I/O instructions

SET WDT DEBUG=LIGHTS		trace IMP status light changes

Remember that you must enable debugging output first before these settings will be effective; refer to the SIMH User’s Guide, “Controlling Debugging,” for more information.

## Communications Devices

### Modem Interface

IMPs and TIPs communicated with each other via leased telephone lines and synchronous modem links.  These modem links were point to point – each IMP/TIP modem communicated with exactly one other modem attached to exactly one other IMP/TIP.  SIMH simulates these modem interfaces and allows them to be attached to either a real, physical, serial port on the host computer or to a virtual UDP/IP port.  Either can then be connected to another simh instance running the IMP/TIP software, or even a real IMP or TIP if someone gets one working again, and a network can be assembled.

Simh implements five modem interfaces, MI1 thru MI5.  Initially MI1 thru 3 are enabled and MI4 and 5 are not, however this can be changed with the commands:

SET MIn ENABLED			enable modem line n

SET MIn DISABLED			disable modem line n

A limitation of the original IMP hardware is that the DMC channels used by modem lines 4 and 5 conflict with those used by host interfaces 3 and 4.  Thus it not possible to enable all modem lines and all host interfaces at the same time (see 3.2.1.2).

Modem interfaces implement one parameter which may be explicitly set:

SET MIn BPS=56000

This parameter sets the simulated line speed, in bits per second, for UDP/IP virtual modem connections.

Modem interfaces also implement an interface (local) and a line (remote) loopback feature.  This cause the modem to receive its own transmitted messages, and are analogous to features in the IMP modem hardware.  Loopback may be enabled or disabled with these commands

SET MIn LOOPINTERFACE		enable interface loopback on line n

SET MIn NOLOOPINTERFACE		disable interface loopback

SET MIn LOOPLINE			enable line loopback on line n

SET MIn NOLOOPLINE		disable line loopback

Modem interfaces support the following registers:

name			size		comments

RXPOLL		32		receiver polling interval

RXPEND		 1		receiver waiting for input

RXERR			 1		receiver error flag

RXIEN			 1		receiver interrupt enable

RXIRQ			 1		receiver interrupt request

RXTOT			32		count of total messages received

TXDLY			32		calculated delay before transmitter done

TXIEN			 1		transmitter interrupt enable

TXIRQ			 1		transmitter interrupt request

TXTOT			32		count of total messages transmitted

LINK			32		link number for h316_udp module

BPS			32		simulated line speed for UDP connections

ILOOP			 1		interface (local) loopback enabled

LLOOP			 1		line (remote) loopback enabled

These registers can be viewed with the command

EXAMINE MIn STATE

The modem device implements these debugging flags:

SET MIn DEBUG=WARN		print warnings for unusual conditions

SET MIn DEBUG=IO			trace all modem interface I/O instructions

SET MIn DEBUG=UDP			trace all UDP packets and connections

SET MIn DEBUG=MSG			trace all IMP messages sent or received

Remember that you must enable debugging output first before these settings will be effective; refer to the SIMH User’s Guide, “Controlling Debugging,” for more information.

#### UDP/IP Tunnels

A virtual modem may also be tunneled using UDP/IP to another simh instance.  UDP connections are symmetrical – each virtual modem listens for incoming packets on a port which you define, and transmit outgoing packets to another host and port which you also define.  It’s your responsibility to define the ports appropriately so that the input of one modem is logically connected to the output of another modem.  These connections must be one to one.  Connecting more than one modem output to the same modem input will not cause any problems for SIMH, but the IMP software will become hopelessly confused.

The general form of a virtual modem ATTACH command for UDP connections is like this

ATTACH MIn llll:w.x.y.z:rrrr

This will listen for incoming packets on port “llll”, which should be a decimal number, and will transmit outgoing packets to port “rrrr” on the host with IP address “w.x.y.z”.  The actual port numbers you use are arbitrary; however some care must be used to avoid conflicts with any other network applications.

The UDP attach command also has a few alternative forms; for example

ATTACH MIn 4431:imp.jfcl.com:4432

will listen on port 4431 of the current host, do a DNS lookup to determine the IP of the host “imp.jfcl.com”, and then transmit to port 4432 on that host.  In another example

ATTACH MIn 1201::1202

will listen to port 1201 on the current, local, host and transmit to port 1202 also on the local host.  This is useful when both SIMH instances are running on the same PC.  In this case, you must use different ports for transmitting and receiving.  If they are the same, the modem will transmit to itself!

Either end of the UDP connection may be disconnected with the command

DETACH MIn

#### Physical Serial Tunnels

The command

ATTACH –p MIn COMnn

is reserved for a possible future option to attach a physical serial port to a virtual modem.  This functionality is not currently implemented.

### Host Interface

NOTE

The current Host Interface implementation is presently only a skeleton – just enough code exists to allow the IMP software to run.  In this version, the IMP effectively sees all attached hosts as permanently powered down.  Hopefully the host interface implementation can be completed at some point, just as soon as there is a suitable host emulator for it to talk to.

The IMP used the host interface to connect the H316 to an Arpanet host mainframe.  This could be a PDP-10, an SDS Sigma 7, a CDC 6600, an IBM 360, or any one of many other machines.  Each IMP could support up to four host interface cards; in simh these are the devices HI1 thru HI4.  Initially HI1 and 2 are enabled and HI3 and 4 are not, however this can be changed with the commands:

SET HIn ENABLED			enable host interface n

SET HIn DISABLED			disable host interface n

Host interfaces support the following registers:

name			size		comments

POLL			32		host polling interval

RXIEN			 1		receiver interrupt enable

RXIRQ			 1		receiver interrupt request

RXTOT			32		count of total messages received

TXIEN			 1		transmitter interrupt enable

TXIRQ			 1		transmitter interrupt request

TXTOT			32		count of total messages transmitted

READY			 1		host ready

FULL			 1		host buffer full

ERROR			 1		host error

LLOOP			 1		local loopback enabled

These registers can be viewed with the command

EXAMINE HIn STATE

The modem device implements these debugging flags:

SET HIn DEBUG=IO			trace all host interface I/O instructions

Remember that you must enable debugging output first before these settings will be effective; refer to the SIMH User’s Guide, “Controlling Debugging,” for more information.

#### UDP/IP Tunnels

In simh host interfaces are simulated using UDP connections to other simh instances that simulate the host mainframe and run the Arpanet host operating system.  The ATTACH command is used to connect a host interface to this other simh instance

ATTACH HIn tba

The host connection may be broken with the command

DETACH HIn

# Summary

## Additional Devices

Table 1 lists the IMP/TIP specific devices added to simh along with their I/O address, interrupt number and vector, and DMC channel assignment.  Note that all the IMP/TIP devices use the H316 extended interrupts and so interrupt 1, for example, corresponds to A bit 1 in the SMK 120 instruction.  Table 2 shows the extended interrupt mask associated with these devices.

| DEVICE |  | INTERRUPT |  | DMC10 | DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| Name | Address8 | Number10 | Vector8 |  |  |
| WDT | 26 |  | 000062 |  | Watch Dog Timer |
| RTC | 40 | 15 | 000102 |  | Real Time Clock |
| IMP | 41 | 16 | 000103 |  | Task Switch |
| IMP | 42 |  |  |  | MLC support |
| HI4 | 50 | 9 | 000074 | 10 | Host Interface #4 (RX) |
|  |  | 4 | 000067 | 5 | Host Interface #4 (TX) |
| HI3 | 51 | 10 | 000075 | 16 | Host Interface #3 (RX) |
|  |  | 5 | 000070 | 15 | Host Interface #3 (TX) |
| HI2 | 60 | 14 | 000101 | 14 | Host Interface #2 (RX) |
|  |  | 12 | 000077 | 12 | Host Interface #2 (TX) |
| HI1 | 70 | 13 | 000100 | 13 | Host Interface #1 (RX) |
|  |  | 11 | 000076 | 11 | Host Interface #1 (TX) |
| MI1 | 71 | 1 | 000064 | 1 | Modem Interface #1 (RX) |
|  |  | 6 | 000071 | 6 | Modem Interface #1 (TX) |
| MI2 | 72 | 2 | 000065 | 2 | Modem Interface #2 (RX) |
|  |  | 7 | 000072 | 7 | Modem Interface #2 (TX) |
| MI3 | 73 | 3 | 000066 | 3 | Modem Interface #3 (RX) |
|  |  | 8 | 000073 | 8 | Modem Interface #3 (TX) |
| MI4Error: Reference source not found | 74 | 4 | 000067 | 4 | Modem Interface #4 (RX) |
|  |  | 9 | 000074 | 9 | Modem Interface #4 (TX) |
| MI5Error: Reference source not found | 75 | 5 | 000070 | 5 | Modem Interface #5 (RX) |
|  |  | 10 | 000075 | 10 | Modem Interface #5 (TX) |

Table 1 - IMP/TIP Devices

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M1RX | M2RX | M3RX | M4RX | M5RX | M1TX | M2TX | M3TX | M4TX | M5TX | H1TX | H2TX | H1RX | H2RX | RTC | TASK |
|  |  |  | H4TX | H3TX |  |  |  | H4RX | H3RX |  |  |  |  |  |  |

Table 2 - Extended Interrupt Mask

## Modem Interface I/O Instructions

Table 3 summarizes the modem interface I/O instructions.  In this table, “n” represents the modem number, (1 thru 5), and “dd” is the device I/O address assigned to that modem (718 thru 758).

| Mnemonic | Opcode | Operation |
| --- | --- | --- |
| MnOUT | 0300dd | start modem output |
| MnUNXP | 0301dd | un-cross patch modem |
| MnLXP | 0302dd | enable line cross patch |
| MnIXP | 0303dd | enable interface cross patch |
| MnIN | 0304dd | start modem input |
| MnERR | 0704dd | skip on modem error |

Table 3 – Modem Instructions

## Host Interface  I/O Instructions

Table 4 summarizes the host interface I/O instructions.   In this table, “n” represents the host number, (1 thru 4), and “dd” is the device I/O address assigned to that host (508, 518, 608, or 708).

| Mnemonic | Opcode | Operation |
| --- | --- | --- |
| HnROUT | 0300dd | start regular output to host |
| HnIN | 0301dd | start host input |
| HnFOUT | 0302dd | start host final output |
| HnXP | 0303dd | cross patch host |
| HnUNXP | 0304dd | un-cross patch host |
| HnENAB | 0305dd | enable host interface |
| HnERR | 0700dd | skip on host error |
| HnRDY | 0701dd | skip on host ready |
| HnEOM | 0702dd | skip on end of host message |
| HnFULL | 0705dd | skip on host buffer full |

Table 4 - Host Instructions

## Other I/O Instructions

Table 5 summarizes the additional miscellaneous I/O instructions.

| Mnemonic | Opcode | Device | Operation |
| --- | --- | --- | --- |
| SMK 120 | 170120 | CPU | set extended interrupt mask |
|  | 030026 | WDT | reset watch dog timer |
|  | 170026 | WDT | set status lights |
| AMI512 | 070026 | WDT | skip if this machine is a DDP-516 |
| CLKON | 030040 | RTC | enable RTC |
| CLKOFF | 031040 | RTC | disable RTC |
| RDCLOK | 131040 | RTC | read RTC count and always skip |
| TASK | 030041 | IMP | cause task switch interrupt |
| RDIMPN | 131041 | IMP | read IMP number and skip |
| AMIMLC | 070042 | IMP | skip if this machine is a multi-line controller |

Table 5 - Other Instructions

The modem 4 interrupt and DMC conflict with host 4 – only one of the two may be active.

The modem 5 interrupt and DMC conflict with host 3 – only one of the two may be active.

On simh this instruction is a NOP and never skips (simh simulates an H316, not the DDP-516).

“Multi-line controller” was the official name for the TIP.  MLC support is not implemented and this instruction is currently a NOP and never skips.