MCS-51 Microcontroller Family Bootstrap Program

BBBBBB    OOOOO    OOOOO  TTTTTTTT       5555555   11

BB   BB  OO   OO  OO   OO    TT          55       111

BB   BB  OO   OO  OO   OO    TT          55        11

BBBBBB   OO   OO  OO   OO    TT    ====  555555    11

BB   BB  OO   OO  OO   OO    TT               55   11

BB   BB  OO   OO  OO   OO    TT               55   11

BBBBBB    OOOOO    OOOOO     TT          555555   1111

U S E R ' S    M A N U A L

Version  1.1

December 15, 2002

copyright (c) 1996, 2002  by  W.W. Heinz

TABLE OF CONTENTS

-----------------

I.    Introduction

II.   BOOT-51 Installation

II.1   Files

II.2   Customizing BOOT-51

II.3   Tailoring the Cables

II.4   Design of MCS-51 Evaluation Boards

III.  BOOT-51 Operation

III.1  Interactive Operation

III.2  Batch File Operation under MS-DOS

III.3  DOS Upload Utilities

III.4  Shell Script Operation under Linux

III.5  The BLINK Program

IV.   Troubleshooting

IV.1   General Trouble

IV.2   Trouble under MS-DOS

IV.3   Trouble under Linux

Appendix A:  CUSTOMIZ Error Messages

A.1    Parameter Errors

A.2    Baudrate Errors

A.3    Runtime Errors

Appendix B:  RESET51 Error Messages

Appendix C:  BOOT-51 Error Messages

Appendix D:  Trademarks

Appendix E:  Tables of Baudrates

I. Introduction

---------------

Usually an 8051 application program is not running correctly from scratch,

and it is not quite easy to debug it without a suitable test equipment.

It is very inconvenient, to burn the program into an EPROM after every

change. Instead it would be nice to load the program into the external

RAM of the target board and then start it. This would dramatically speed

up the turn-around cycles. However, three problems must be solved first:

1. There must be a suitable interface to transmit the application

program from the host (PC) to the target (8051) system.

2. The target board needs a suitable firmware that can receive a

program from the host computer, store it in the external RAM,

and finally execute it.

3. Once loaded, the application program must be executable, although

it is stored in the external RAM, which is usually mapped into the

XDATA address space.

(Programs can only be executed in the CODE address space!)

The corresponding three solutions are:

1. an RS-232 interface between the PC and the 8051 board

2. a bootstrap program

3. external RAM that is mapped into both the CODE and XDATA address space

Nearly all evaluation boards for 8051 derivatives are meeting requirements

1 and 3. In most cases, it is easy to establish a memory map according to

requirement 3, even on target boards that do not originally support it.

(See chapter "II.4 Design of MCS-51 Evaluation Boards".)

Requirement 2 can be met by burning a customized version of BOOT-51 into

the EPROM of the target board. After system reset, it can receive an

Intel-HEX file over the serial interface, store it in the external RAM,

and finally jump to the program start address.

BOOT-51 itself doesn't need any external RAM, and requires only 1 kB of

EPROM. In principle, it is not restricted to a PC-based host system, but

presently only the host platforms MS-DOS, Windows 9x, and Linux (i386)

are supported.

Sure, a bootstrap program cannot replace a target debugger, but it's a start.

II. BOOT-51 Installation

------------------------

The BOOT-51 support files are usually installed on the host system together

with the latest ASEM-51 version. Of course BOOT-51 must be installed on the

MCS-51 target system, too.

Before burning BOOT-51 into an EPROM, it must be customized for the target

board, and one or two cables must be tailored for local requirements.

If the design of the target system is not yet finished, some basic knowledge,

concerning the general design of MCS-51 evaluation boards, might be helpful

to meet the hardware requirements from the beginning.

II.1 Files

----------

The BOOT-51 package for DOS/Windows consists of the following files:

BOOT51.DOC          BOOT-51 User's Manual, ASCII format

BOOT51.HTM          index file of the BOOT-51 documentation, HTML format

*.HTM          further pages of the HTML documentation

*.GIF          GIF images referenced by HTML pages

BOOT51.A51          BOOT-51 assembler source (for ASEM-51 V1.3 and up)

CUSTOMIZ.EXE          BOOT-51 customization utility

BOOT.BAT          batch file for application program upload

UPLOAD.BAT          called by BOOT.BAT only

COMPORT.EXE          setup utility for PC serial ports

RESET51.EXE          program to reset the target system via PC ports

SLEEP.EXE          program to wait for the reset recovery time

BLINK.A51          sample test program for BOOT-51

The BOOT-51 package for Linux consists of the following files:

boot51.doc          BOOT-51 User's Manual, ASCII format

boot51.htm          index file of the BOOT-51 documentation, HTML format

*.htm          further pages of the HTML documentation

*.gif          GIF images referenced by HTML pages

boot51.a51          BOOT-51 assembler source (for ASEM-51 V1.3 and up)

customiz            BOOT-51 customization utility

customiz.1          man-page for customiz

boot                shell script for application program upload

boot.1              man-page for boot

upload              called by boot only (generic version)

upload.new          "new" upload (optimized for stty 2.0 or later)

reset51             program to reset the target system via PC ports

reset51.1           man-page for reset51

blink.a51           sample test program for BOOT-51

II.2 Customizing BOOT-51

------------------------

BOOT-51 must be customized for the target board, before it is ready for

assembly. This is performed with an include file boot51.inc, containing

the required configuration data. boot51.inc is generated with the BOOT-51

Customization Utility CUSTOMIZ.

There are two major groups of configuration parameters:

-  baudrate generation

-  memory addresses

CUSTOMIZ is invoked as shown below:

customiz <baud> [<timer> [<prec> [<clock> [<start> [<user>]]]]]

The first four parameters are for baudrate generation, and the last two

parameters are memory addresses. Parameters in brackets are optional.

If CUSTOMIZ is invoked without parameters, it displays a help screen.

<baud>     is the desired nominal baudrate (in Baud), BOOT-51 should use

for communication with the host system.

<timer>    is the on-chip timer or baudrate generator that is used for

baudrate generation. Legal values are:

T1 = timer 1, available on nearly all 8051 derivatives (default)

T2 = timer 2, available on 8052/32 and all true compatibles

BG = internal baudrate generator on 80C515/35 and 80C517/37

BA = internal baudrate generator on 80C515A and 80C517A

D1 = timer 1 with clock/12 or clock/4 prescaler on 80C320

<prec>     is the required relative precision of the baudrate in %.

The default precision is 1.0 %.

<clock>    is the system clock frequency in MHz. (11.0592 MHz default)

<start>    is the start address of BOOT-51 on the target system in hex

representation. (default is 0000H)

<user>     is the usual hex start address of the application programs

in the external memory. (default is 8000H)

Baudrate:

---------

For the serial interface, the standard UART is used that is available on

almost every 8051 derivative. BOOT-51 is always running the UART in Mode 1,

that is asynchronous mode, 8 data bits, 1 stop bit, no parity.

However, depending on the derivative, there may be various timers or

prescalers that can be used for baudrate generation.

If the desired baudrate cannot be generated or the precision is too bad,

CUSTOMIZ will display a corresponding error message. In this case, try

another baudrate generator that is also available on your derivative.

If this fails, too, try another baudrate that may also do, or think of

using another oscillator crystal that suits better for the task.

You should play around with CUSTOMIZ, to explore how to get best results.

The tables in Appendix E show the precisions of the most frequently used

baudrates that can be generated with the supported timers and baudrate

generators, for some standard clock frequencies.

In most cases baudrate deviations of 2 to 3 % are no problem for the

communication with a PC. Whenever in doubt, try it!

BOOT-51 can receive data at very high baudrates without the need for a

handshake protocol. Output is done more slowly with short pauses between

the characters, to avoid a data overrun of the PC UART. The maximum

effective output speed of BOOT-51 is corresponding to 9600 Baud

(max. 1 character per ms).

In general, speed is rather a problem for the PC than for the micro.

Example 1:   customiz 9600 BG 0.1 12

----------

will generate an include file for BOOT-51 that configures it

for 9600 Baud, generated with the internal baudrate generator

of a SAB80C535 or SAB80C537 that is clocked with 12.0 MHz.

The required precision is 0.1 %.

The bootstrap and application program start addresses are

set to their default values.

Addresses:

----------

<start>:

An 8051 starts program execution at CODE address 0000H after reset.

Thus there must always be ROM at CODE address 0000H to execute the first

instructions. On most 8051 boards there is EPROM in the range 0000H-7FFFH,

and RAM in the range 8000H-FFFFH. However, the final application programs

will usually start at address 0000H, and the 8051 interrupt addresses start

at address 0003H. If application programs can be loaded into RAM at 8000H,

this is normally not quite realistic for testing.

Hence it would be nice to have CODE/XDATA-RAM at address 0000H to load and

test application programs. That is why some evaluation boards (e.g. the Keil

MCB-517) remap the EPROM from address 0000H to 8000H, and the RAM from 8000H

to 0000H, when the first memory access to a CODE address > 7FFFH occurs.

To support this, BOOT-51 starts with a long jump to its entry point, to

eventually remap the memory. If you own a corresponding evaluation board,

specify the <start> address of BOOT-51 to the location, where it resides

after memory remapping!

<user>:

In principle, BOOT-51 can load an application program at any location in

the external RAM and start it there. However, the interrupt addresses

should be redirected to the location, where application programs usually

start, e.g. the start address of the external RAM.

Hence the <user> address should point to the location, where the CODE/XDATA-

RAM starts. If it starts at 8000H, interrupt 0003H is redirected to 8003H,

interrupt 000BH to 800BH, and so on.

If the CODE/XDATA-RAM starts at 0000H (after remapping), the <user> address

has no practical meaning and can be left default.

Example 2:   customiz 19200 T2 1 11.0592 0 C000

----------

will generate an include file for BOOT-51 that configures it

for 19200 Baud, generated with timer 2 of an 80C52 or 80C32

that is clocked with 11.0592 MHz. The required precision is 1 %.

BOOT-51 is located at address 0000H in the EPROM, and the

interrupt addresses are redirected to location 0C000H.

If CUSTOMIZ aborts with an error message, it returns exit code 1 on para-

meter- and baudrate-errors, and exit code 2 on fatal runtime errors.

When CUSTOMIZ terminates without error message, it returns exit code 0, and

a customization header file boot51.inc should be present in the default

directory. With this file, BOOT-51 can be assembled as usual with

ASEM BOOT51                           (DOS)

asem boot51.a51                       (Linux)

Remember that BOOT-51 requires ASEM-51 V1.3 or later!

Now there should be an Intel-HEX file boot51.hex, which may serve directly

as an input for the EPROM programmer. We may also convert it to a binary

image file with the HEXBIN utility:

HEXBIN BOOT51/LENGTH:4000              (DOS)

hexbin -l 4000 boot51.hex              (Linux)

would generate a binary image file boot51.bin for a 16 kB EPROM (27C128).

(Note: Only 1 kB of EPROM is required for the BOOT-51 program code.)

Once in an EPROM, BOOT-51 should be ready to run on the target system.

II.3 Tailoring the Cables

-------------------------

Application programs are uploaded to the target system over a serial

interface. The serial ports of the PC may have 9-pin or 25-pin male

D-Shell connectors. Suitable RS-232 cables for the MCS-51 board can

be tailored as follows:

RS-232 cable with 25-pin female D-Shell connector:

--------------------------------------------------

PC serial port:                                  MCS-51 target system:

pin

Tx   2  o----------------------------------------->  RXD

Rx   3  o----------------------------------------->  TXD

RTS   4  o-----+

|

CTS   5  o-----+

DSR   6  o-----+

|

DCD   8  o-----+

|

DTR  20  o-----+----------------------------------->  Reset

ground   7  o----------------------------------------->  ground

Shortening the modem control signals is absolutely required for batch

operation under MS-DOS! If the CTS-, DSR-, or DCD-input is not set to

logic low (+12V), the PC will hang during character output to the serial

port.

For shell script operation under Linux, only the Tx-, Rx-, and ground

lines are required, and the handshake inputs may be left open.

The same is true for interactive operation with most terminal emulation

programs. (For further information refer to the documentation of your

terminal emulation program.)

If there is a spare RS-232 line receiver on your MCS-51 board, it can be

employed as a reset input! In this case, the target system can be reset

in batch (or script) files with the reset program provided, and from terminal

emulation programs (by dropping the DTR output with the hang-up function).

RS-232 cable with 9-pin female D-Shell connector:

-------------------------------------------------

PC serial port:                                  MCS-51 target system:

pin

Rx   2  o----------------------------------------->  TXD

Tx   3  o----------------------------------------->  RXD

RTS   7  o-----+

|

CTS   8  o-----+

DCD   1  o-----+

|

DTR   4  o-----+----------------------------------->  Reset

|

DSR   6  o-----+

ground   5  o----------------------------------------->  ground

If you own a 9/25-pin mouse adaptor with all pins connected, the 9-pin

version of the cable should be preferred!

If your MCS-51 board has no spare RS-232 line receivers, but a TTL reset

input (or you can establish one), the target system can also be reset in

batch (or script) files over a printer port of the PC.

For this, a suitable reset cable must be tailored as shown below:

Reset cable with 25-pin male D-Shell connector:

-----------------------------------------------

PC printer port:                                 MCS-51 target system:

pin

D0   2  o----------------------------------------->  Reset

_____

D1   3  o----------------------------------------->  Reset

ground  25  o----------------------------------------->  ground

In practice, only one of the two reset lines will be required.

When the reset program is invoked, a positive reset pulse appears

at D0 (8031, 8032), and a negative pulse at D1 (80C535, 80C537).

For detailed information on the reset program, refer to chapters

"III.2 Batch File Operation under MS-DOS", and

"III.4 Shell Script Operation under Linux".

II.4 Design of MCS-51 Evaluation Boards

---------------------------------------

When developing embedded systems, debugging aspects should be taken into

account as soon as possible. That is why MCS-51 prototype boards should

always be armed with a download RAM in the hardware design phase, even if

the serial model (probably) doesn't need an external RAM!

A $3 RAM can save a $3000 in-circuit emulator and weeks of trouble!

Before discussing the basic concepts of program download, it is essential

to understand the memory organization of the MCS-51 architecture.

First of all, there is an 8-bit address space for internal RAM. Parts of

it are directly addressable (DATA space), indirectly addressable (IDATA

space), or bit-addressable (BIT space). These address spaces are fully or

partly overlapping. Furthermore, the four register banks and the Special

Function Registers (SFR) are mapped into this internal address space. This

is hard to understand for newbies, but plays no role for program download.

Aside of the internal memory, the MCS-51 architecture provides a bus

interface for external memory. There are two separate 16-bit address spaces

for 64K of program memory (ROM) and 64K of data memory (RAM). Program memory

(CODE space) can only be read with the -PSEN bus signal, and data memory

(XDATA space) can be read and written with the -RD and -WR bus signals.

The -PSEN signal becomes active during instruction fetch cycles and MOVC

instructions. The -RD and -WR signals become active during read and write

operations with MOVX instructions.

The obvious problem is: a downloaded program can only be stored in external

RAM, mapped into the XDATA space, whereas program code can only be executed

from memory in the CODE space.

There must be a way to execute a downloaded program, although it is stored

in external RAM! Fortunately, there is a simple trick to solve this:

The -OE input of the (usually static) RAM must be driven by a logic AND of

the -RD and -PSEN signals of the MCU, rather than by -RD alone. That's all!

Then the external RAM can be read and written with MOVX instructions (using

-RD and -WR), and program code can be executed from it (using -PSEN).

For example, a typical 8051 evaluation board could be designed as follows:

A 32K EPROM (e.g. 27C256) is mapped into the CODE space from address 0000H

to 7FFFH, containing BOOT-51 at address 0000H.

A 32K static RAM (e.g. 62256) is mapped into both the CODE and XDATA spaces

from address 8000H to FFFFH.

After reset, the MCU starts program execution at address 0000H, and BOOT-51

is running, waiting for commands. When it receives an UPLOAD command from

the host computer, it reads an Intel-HEX file and stores it at its start

address in the external RAM (e.g. 8000H). If BOOT-51 receives a GO TO 8000

command, it jumps to the specified address 8000H, and the downloaded program

is running.

A memory map like this can be obtained with a minimum of hardware:

The -CE (chip enable) input of the EPROM must be driven by the A15 signal

(P2.7) of the MCU, and -CE of the RAM with the inverted A15 signal

respectively, to avoid bus conflicts in the CODE space.

The -OE (output enable) input of the EPROM must be driven by the -PSEN

signal of the MCU, as usual for CODE memory.

The -WE (write enable) input of the RAM must be driven by the -WR signal

(P3.6) of the MCU, as usual for XDATA memory.

Only the -OE (output enable) input of the RAM must be driven by that famous

logical AND of the -RD and -PSEN signals of the MCU, to map the RAM into

both the XDATA and CODE space. That's it!

Because this is so simple, cheap and (in contrast to many in-circuit

emulators) absolutely reliable, most MCS-51 family evaluation boards are

working like this or similar.

In most cases, it is easy to establish a suitable memory map, even on

target boards that do not originally support it.

If there is no spare AND gate, it may also do, to simply connect the -OE

input of the RAM to the -PSEN signal of the MCU only. In this case, the RAM

can still be written, but no longer be read with MOVX instructions. Since

program code can also be executed from it, this minimized version can still

be used as download RAM, but no longer as external data RAM.

(In cases of emergency, it can be read with MOVC instructions, however!)

III. BOOT-51 Operation

----------------------

When BOOT-51 is customized correctly, and the target board is connected to

the host PC with the cable(s) described above, we can start to upload and

run application programs. There are two possible modes of operation:

- interactive operation

- batch operation

III.1 Interactive Operation

---------------------------

BOOT-51 implements a simple ASCII user interface for serial communication.

The bootstrap program can be operated interactively with a terminal emulation

program running on the PC (e.g. TELIX under MS-DOS, or minicom under Linux).

To perform this, start your terminal emulation program, and configure it for

the correct baudrate, 8 data bits, 1 stop bit, no parity, and no handshake.

Then reset the MCS-51 target system.

(If you have established an RS-232 reset line, the target system can usually

be reset by dropping the RS-232 DTR output with the "hang-up" function!)

Now the BOOT-51 sign-on message should appear on your screen:

BOOT-51  V1.1           Copyright (c) 2002 by W.W. Heinz

>

The '>' character indicates that the BOOT-51 command line interpreter is

ready for input. It implements exactly two commands:

1. The UPLOAD command:

First press the 'U', and then the return key.

Now the upload prompt ':' should be displayed. BOOT-51 is now ready to

receive an application program in Intel-HEX format. Send an Intel-HEX

file with the ASCII upload function of your terminal emulation program.

(Be sure that it terminates every line with a CR and/or LF character!)

Now the received Intel-HEX records are loaded to their start addresses

specified in the record headers. If an EOF record is received, the upload

is terminated and the command prompt '>' is displayed again. Now the

command line interpreter is ready for the next command.

When a format error is detected in the Intel-HEX file, BOOT-51 waits

until no more characters are received for about two seconds, and then

aborts with a corresponding error message.  (see Appendix C)

2. The GO TO command:

To start an application program at address 8000H, enter "G 8000" and

press return. BOOT-51 sets now all MCU registers to reset conditions

(as far as possible), and jumps to the start address specified.

Now the previously loaded application program should be running!

Any desired number of Intel-HEX files may be loaded, before entering the

'G' command. The commands may be entered in upper or lower case letters.

Interactive operation should be preferred, if the application program

performs serial I/O. This can easily be supervised with the terminal

emulation program. In interactive mode, only an RS-232 reset output is

applicable. (A printer port reset cable would be of little use!)

III.2 Batch File Operation under MS-DOS

---------------------------------------

If serial I/O is only used for program upload, batch file operation may be

the most convenient way of testing with the shortest turn-around cycles.

The sample batch job below, shows the principles of operating BOOT-51 from

a PC with MS-DOS batch commands:

MODE COM2:9600,N,8,1,P

RESET51 LPT1

SLEEP 9600

ASEM %1

ECHO U >COM2

COPY %1.HEX COM2

ECHO G 8000 >COM2

In this example, serial I/O is done over serial port COM2, and the target

system can be reset over printer port LPT1.

First of all, the serial port COM2 is initialized to the desired baudrate

of 9600 Baud, 8 data bits, 1 stop bit, and no parity with the MODE utility.

Then the RESET51 program provided is executed to reset the target system via

printer port LPT1. After that, the SLEEP program waits until BOOT-51 has

fully output its sign-on message at 9600 Baud and is ready for a command.

To give the MCS-51 board some more time to recover from reset, ASEM-51 is

now invoked to assemble the application program.

When finished, the BOOT-51 upload command 'U' is ECHOed to COM2, and the

Intel-HEX file is also simply copied to the serial port.

Finally the application program is started at address 8000H with the

command "ECHO G 8000 >COM2".

To test an application program MYPROG.A51 with the above batch job (stored

in a file TEST51.BAT), simply type

TEST51 MYPROG

at the DOS prompt, and see what you get.

Of course this was only a minimized example! In general you will not only

have to modify serial port, baudrate, reset port, and start address for

your local requirements, but also to do something for plausibility checking

and error handling.

For this, the more sophisticated batch file BOOT.BAT has been provided.

It allows an easy change of all configuration parameters with any ASCII

editor. For further information see the commentary inside BOOT.BAT.

In principle, BOOT.BAT contains only those configuration data!

(The actual work is done by another batch file UPLOAD.BAT provided,

which is invoked by BOOT.BAT.)

A program MYPROG.A51 can now be assembled, uploaded and started with

BOOT MYPROG

III.3 DOS Upload Utilities

--------------------------

The BOOT-51 package for DOS/Windows contains the utilities COMPORT.EXE,

RESET51.EXE, and SLEEP.EXE, which are usually invoked by UPLOAD.BAT, but

can also be used separately.

COMPORT

-------

sets <baudrate> and data format of a PC serial <port>:

COMPORT <port> <baudrate> [<parity> [<databits> [<stopbits>]]]

Legal parameter values are:

<port>:        AUX, COM1, COM2, COM3, COM4

<baudrate>:    1.66 < baudrate < 121000

<parity>:      N, O, E, M, S     (default: N)

<databits>:    5, 6, 7, 8        (default: 8)

<stopbits>:    1, 1.5, 2         (default: 1)

The port names AUX and COM1 are equivalent.

COMPORT allows to set very low, very high, and very odd baudrates, which

can be generated by the 16C450 UART of the PC (and the 8051 board), but

are not supported by the PC BIOS or the MS-DOS MODE command.

Baudrates that cannot be generated exactly are accepted as long as the

deviation doesn't exceed 5.0 %. COMPORT displays the "real" baudrate and

the relative deviation (if any) from the nominal value.

The parity can optionally be set to N (none), O (odd), E (even), M (mark),

or S (space). Furthermore, the data format can be set to 5, 6, 7, or 8

data bits, and 1, 1.5, or 2 stop bits.

Default settings are: no parity, 8 data bits and 1 stop bit.

Note:  only 6, 7 or 8 data bits may be combined with 1 or 2 stop bits.

5 data bits are only possible with 1.5 stop bits!

Example 1:      COMPORT COM4 12800

----------

Sets the serial port COM4 to 12800 Baud, no parity,

8 data bits, and 1 stop bit.

RESET51

-------

forms a reset pulse of the duration <pulsewidth> at the D0 and D1 outputs

of a PC printer <port>, or at the DTR output of a serial <port>:

RESET51 <port> [<pulsewidth>]

Legal <port> values are:  PRN, LPT1, LPT2, LPT3,

AUX, COM1, COM2, COM3, COM4,

NUL

PRN and LPT1 are equivalent, and so are AUX and COM1.

The optional parameter <pulsewidth> is the duration of the reset pulse in ms.

Legal values are:  1 <= <pulsewidth> <= 65535.  (default is 50)

When invoked without parameters, a help screen is displayed.

In case of error, RESET51 returns the ERRORLEVEL 1, 0 otherwise.

When executed, RESET51 forms a positive pulse at D0 (pin 2), and a negative

pulse at D1 (pin 3) of a PC printer port, or a "high" pulse (-12 V !!!) at

the DTR output of a serial port respectively. If the NUL device is specified

as <port>, RESET51 delays only for <pulsewidth> ms.

If the default pulse width of 50 ms is too short to reset the MCS-51 board,

specify a greater pulse width.

Example 2:      RESET51 LPT2 500

----------

Applies a reset signal to the D0 and D1 outputs of the

printer port LPT2 for 500 ms.

RESET51 is running under MS-DOS, and in the DOS-Boxes of Windows 3.1x and 9x.

Windows ME does no longer provide a DOS-Box.

RESET51 is not running under Windows NT, 2000, and XP!

For batch file operation, a reset line is highly recommended! If you don't

have it, you will always have to press the reset button of the target system,

before invoking BOOT.BAT. Since this may easily be forgotten, it is better

to automate this step.

SLEEP

-----

waits until BOOT-51 has output <chars> characters of its sign-on message

at a given <baudrate>:

SLEEP <baudrate> [<chars>]

SLEEP is a simple delay program, which is waiting as long as it takes to

output <chars> characters with the specified <baudrate>. However, SLEEP

takes into account, that BOOT-51 will never send more than one character

per ms, even at very high baudrates.

The default value of <chars> is 70, which is slightly more than the total

length of the BOOT-51 sign-on message.

Example 3:      SLEEP 300

----------

Waits until BOOT-51 has fully output its sign-on message

at 300 Baud.

III.4 Shell Script Operation under Linux

----------------------------------------

If serial I/O is only used for program upload, shell script operation may be

the most convenient way of testing with the shortest turn-around cycles.

The sample script file below, shows the principles of operating BOOT-51 from

a PC, running under Linux, with shell commands:

#!/bin/sh

stty 9600 sane clocal -crtscts -hupcl </dev/ttyS1      # until stty 1.16

# stty -F /dev/ttyS1 9600 sane clocal -crtscts -hupcl  # stty 2.0 or later

reset51 /dev/lp0

sleep 1

asem $1.a51

echo "U" >/dev/ttyS1

cp $1.hex /dev/ttyS1

echo "G 8000" >/dev/ttyS1

In this example, serial I/O is done over serial port /dev/ttyS1, and the

target system can be reset over printer port /dev/lp0.

First of all, the serial port /dev/ttyS1 is initialized to the desired

baudrate of 9600 Baud, 8 data bits, 1 stop bit, no parity and no handshake

with the stty command. (If your stty is version 2.0 or later, better use

the new command syntax with the -F option, as shown in commentary!)

Then the reset51 program provided is executed to reset the target system via

printer port /dev/lp0. After that, the sleep command waits until BOOT-51 has

recovered from reset, fully output its sign-on message, and is ready for a

command.

Now ASEM-51 is invoked to assemble the application program.

When finished, the BOOT-51 upload command 'U' is echoed to /dev/ttyS1, and

the Intel-HEX file is also simply copied to that serial port.

Finally the application program is started at address 8000H with the

command 'echo "G 8000" >/dev/ttyS1'.

To test an application program myprog.a51 with the above shell script

(stored in a file test51), simply type

test51 myprog

at the shell prompt, and see what you get.

Of course this was only a minimized example! In general you will not only

have to modify serial port, baudrate, reset port, and start address for

your local requirements, but also to do something for plausibility checking

and error handling.

For this, the more sophisticated script file boot has been provided.

It allows an easy change of all configuration parameters with any ASCII

editor. For further information see the commentary inside boot.

In principle, boot contains only those configuration data!

(The actual work is done by another script upload provided, which is

invoked by boot. If your stty command is version 2.0 or later, better

use the script file upload.new instead!)

A program myprog.a51 can now be assembled, uploaded and started with

boot myprog

To get the boot script running, ensure that you have full read/write

access to the serial port used as upload device!

If a printer port is employed as reset device, the reset51 program requires

root privileges!  (see below)

Finally, the reset51 program provided, may be worth a detailed explanation:

When invoked, reset51 forms a reset pulse of the duration <pulsewidth> at

the D0 and D1 outputs of a PC printer <port>, or at the DTR output of a

serial <port>:

reset51 /dev/<port> [<pulsewidth>]

Legal <port> values are:  lp0, lp1, lp2,

ttyS0, ttyS1, ttyS2, ttyS3,

cua0, cua1, cua2, cua3,

null

Under Linux with kernels 2.0.x the printer ports are assigned to fixed

I/O base addresses (lp0 = 3BCH, lp1 = 378H, lp2 = 278H).

From kernel 2.2.0, the parport layer has been introduced, providing dynamic

printer port assignment. If your 2.2.x kernel doesn't contain parport layer

support or the /proc file system, you should recompile it respectively.

This should also make PCI bus printer ports available as reset devices.

reset51 can only handle the four standard PC serial ports ttyS0 thru ttyS3.

(However, the boot script described above should also work with non-standard

serial ports, because it operates them with the stty command only!)

The serial ports ttyS0 and cua0 are equivalent, and so are ttyS1 and cua1,

and so on.

If /dev/null is specified as <port>, reset51 delays only for <pulsewidth> ms.

The optional parameter <pulsewidth> is the duration of the reset pulse in ms.

Legal values are:  1 <= <pulsewidth> <= 65535.  (default is 50)

When invoked without parameters, a help screen is displayed.

In case of error, reset51 returns the exit code 1, 0 otherwise.

When executed, reset51 forms a positive pulse at D0 (pin 2), and a negative

pulse at D1 (pin 3) of a PC printer port, or a "high" pulse (-12 V !!!) at

the DTR output of a serial port respectively.

If the default pulse width of 50 ms is too short to reset the MCS-51 board,

specify a greater pulse width.

Since reset51 needs direct access to I/O ports, it requires root privileges!

To make it available for all users, set the owner to "root" and set the

"set-user-id"-bit:

chown root reset51

chmod u+s reset51

For shell script operation, a reset line is highly recommended! If you don't

have it, you will always have to press the reset button of the target system,

before invoking the boot script. Since this may easily be forgotten, it is

better to automate this step.

III.5 The BLINK Program

-----------------------

When BOOT-51 is customized and blown into an EPROM, and the target board

is connected to the host PC, it would be nice to verify, whether the whole

configuration is working together correctly. This can be done with a short

test program that performs a well-defined action when started.

For this purpose the tiny assembly program blink.a51 has been provided.

It simply toggles the logic level of a port pin (roughly) once a second

(great if connected to a LED) to verify, whether all the components in

the chain

assembly -> serial interface -> target system <-> bootstrap program

are working together correctly.

It can be adapted to your requirements with little efforts:

1. If your target system carries a LED that can be switched with a

bit-addressable port pin, please change the BIT symbol LEDPIN

accordingly. Then the program will make the LED blink.

If your LED can only be switched with a non-bit-addressable

port pin (say bit 2 of a port P6), simply replace the statement

"CPL LEDPIN" by "XRL P6,#00000100B" or something like that.

2. Change the program start address START to the location, where

user programs are usually loaded on your target system.

The program code itself is position-independent!

If there is no LED on your target system, connect the port pin to

a volt-meter. This may also do. Aside of P0 and P2 you may use every

port with LEDs or spare outputs. Now the command

boot blink

should reset the MCS-51 board, assemble the test program, upload it to the

target system, and finally start it. If the LED is blinking, everything is

now perfectly installed and ready for daily work.

If not, you should read the next chapter!

IV. Troubleshooting

--------------------

Of course, you have read all the previous chapters carefully (haven't you?),

and you have checked (and double-checked) your hardware, but the damn thing

is not willing to work!

If you don't have an idea what may still be wrong, here are some typical

cases from my own experience:

IV.1 General Trouble

--------------------

These are problems that may occur on all host platforms:

1. The RESET51 program terminates without any error, but the target system

is not reset.

The (reset) cable may be plugged into the wrong (printer) port.

First of all try the other ports.

There may also be something wrong with the cable itself.

Employ the BOOT-51 User's Manual and the circuit diagram of the target

board, and check the cable.

Furthermore, the reset pulse may be too short.

Specify a greater pulse width.

You may also decrease the reset capacitor of the MCS-51 system.

2. The target system is reset all the time.                            _____

When tailoring the reset cable, you may have mixed up the RESET and RESET

lines. Simply interchange them.

3. In batch (or script) mode everything seems to work fine on the PC, but

on the target system you get absolutely nothing.

Try the interactive mode and reset the target system. If nothing happens,

you may have mixed up the receive and transmit lines of the target board.

If you get some garbage, the baudrates and/or serial data formats of the

target system and the PC are probably different.

Configure your terminal emulation program correctly, and change BOOT.BAT

(under MS-DOS) or the boot script (under Linux) accordingly.

(correct data format: 8 data bits, 1 stop bit, no parity)

4. Everything is working fine in batch (or script) mode, but when you try

to upload the same HEX file in interactive mode, BOOT-51 aborts with the

error message "unexpected character".

Check the ASCII upload configuration parameters of your terminal

emulation program. It has to terminate every line with a CR and/or LF

character, and it must not send any pace characters!

Of course it is always best, to send the HEX file exactly as it is.

IV.2 Trouble under MS-DOS

-------------------------

These are problems that are specific to DOS-based host platforms, including

Windows 3.1x. Most of it also applies to Windows 9x systems.

1. When initializing the serial port, the DOS MODE utility aborts with an

error message like:  "function not supported on this computer", or

"invalid parameter", or something like that.

On most PCs, the BIOS doesn't support baudrates greater than 9600 Baud.

MODE itself doesn't support baudrates greater than 19200 Baud.

However, most terminal emulation programs can operate at baudrates of up

to 115200 Baud.

2. The RESET51 program aborts with "port not found".

The PC-BIOS didn't recognize the printer or serial port specified.

Try another port or check your PC hardware.

3. The RESET51 program aborts with "hardware failure".

The PC-BIOS has recognized the specified port, but the data latch

(printer port) or modem control register (serial port) doesn't work.

Try another port, or check the defective port.

4. The PC hangs when data are sent over the serial interface in batch mode.

The RS-232 cable may be plugged into the wrong COM-port.

First of all, try the other ports.

There may also something be wrong with the modem control signals.

(See chapter "II.3 Tailoring the Cables".)

5. It works in interactive mode, but the PC hangs, if data are sent in batch

mode.

There _is_ something wrong with the modem control lines! Employ the

BOOT-51 User's Manual and check the RS-232 cable at the PC side connector.

6. When invoked, the MS-DOS MODE command hangs, or fails to change the

previous baudrate, although it terminates without error.

Remove all networking and communication drivers and TSR programs from

your CONFIG.SYS and AUTOEXEC.BAT files, reboot, and try it again.

There is plenty of quick'n dirty software, doing something strange

with the BIOS interrupts!

IV.3 Trouble under Linux

------------------------

The most popular problems under Linux are permission conflicts and wrong

(or no) configuration. Problems may even change with the version number

of the Linux kernel!

Note that some changes suggested here will lower your systems security

against attackers who have login access to your machine. Direct access

to hardware ports and security do not mix well.

On some distributions changes in the /dev directory might be detected

by periodically run jobs and automatically reverted to the old state.

1. The boot script aborts with the error message

./upload: /dev/ttyS1: Permission denied

(or something like that).

You don't have read/write access to the serial port in use (ttyS1).

If you are the only user of this computer, you can log in as root,

and simply allow read/write operations for all users:

chmod a+rw /dev/ttyS1

If there are more users who have access to the system, better define

a new group called "embedded" (or something) in /etc/group and add all

users who need access to this port. Finally do

chgrp embedded /dev/ttyS1

chmod o-rw,ug+rw /dev/ttyS1

2. The boot script hangs on any attempt to output an Intel-HEX file to

a particular serial port.

There may be an I/O address or interrupt conflict, or the serial ports

are not configured correctly, or not at all.

Make sure that every serial port uses a base I/O address and an IRQ

that does not conflict with any other hardware installed in your system.

(For example, port ttyS3 at 0x2e8 conflicts with an 8514 graphics card!)

Check the BIOS setup menu for on-board peripherals, as well as the jumper

settings of all the legacy (ISA) boards.

Watch the console output when Linux is booting. There are messages like

ttyS00 at 0x03f8 (irq = 4) is a 16550A

A log file is usually kept in /var/log. The file name depends on the

Linux distribution and version. Look for boot.msg, bootlog, or something.

Linux doesn't really auto-probe the serial ports. It assumes standard

port addresses and IRQ numbers. If your hardware doesn't match this

default configuration, it will not work. In this case you have to

configure your serial ports manually with the setserial command, e.g.

setserial -v /dev/ttyS0 port 0x3f8 irq  4 autoconfig

setserial -v /dev/ttyS1 port 0x2f8 irq  3 autoconfig

setserial -v /dev/ttyS2 port 0x3e8 irq  9 autoconfig

setserial -v /dev/ttyS3 port 0x2e8 irq 10 autoconfig

Unfortunately the best place where to put these commands, in order to

execute them during system boot, depends on the Linux distribution.

For most Linux distributions, /etc/init.d/serial is a good choice!

SuSE distributions kept it in /sbin/init.d/serial until version 7.0.

For detailed information on serial port configuration refer to the

setserial man-pages and the Serial-HOWTO.

3. The boot script aborts with an error message like

resetting target system ...

@@@@@ access denied: /dev/lp0 @@@@@

on a system with a 2.2.x (or later) kernel, or with

resetting target system ...

@@@@@ no root privilege @@@@@

on a system with a 2.0.x (or earlier) kernel.

The reset51 program needs root privileges for direct access to I/O ports!

If you are the only user of this computer, you can log in as root,

and simply allow to run reset51 as root for all users:

chown root reset51

chmod a+x,u+s reset51

Once again a warning: don't do this, if your system should have a higher

level of security! If there are more users who have access to the system,

better define a special group "embedded" (or something) in /etc/group and

add all users who need to run reset51. Then disable execution for all

other users:

chown root reset51

chgrp embedded reset51

chmod o-x,ug+x,u+s reset51

This may considerably improve your system security.

4. The boot script aborts with the error message

resetting target system ...

@@@@@ hardware failure: lp0 @@@@@

although there are two perfectly working printer ports in your PC.

The reason may be that you run a 2.0.x (or earlier) Linux kernel,

which assigns fixed I/O base addresses to the printer port devices:

there is no printer port with base address 0x3bc in your system!

Try /dev/lp1 (0x378) and /dev/lp2 (0x278) as reset devices, and

change your boot script accordingly.

5. The boot script aborts with the error message

resetting target system ...

@@@@@ port not found: /dev/lp2 @@@@@

on a PC with two printer ports and a 2.2.x (or later) kernel.

There is no entry for /dev/lp2 in /proc/parport/2!

From version 2.2.0 the Linux kernel provides the parport layer for

dynamic printer port assignment, as known from the PC-BIOS.

If you have recently updated from a 2.0.x (or earlier) kernel, your

previous printer port /dev/lp2 may now simply be called /dev/lp1 in

a 2.2.x (or later) system.

Try /dev/lp1 as reset device, and change your boot script accordingly.

If this doesn't apply, make sure that

- the /proc file system is configured and mounted

- the kernel contains the kernel module loader kmod

- the kernel is configured for parport support

- the kernel modules lp, parport, and parport_pc are loaded

- all printer ports are configured in /etc/modules.conf, e.g.

options parport_pc io=0x378,0x278 irq=7,none

Configure and recompile your kernel respectively, if necessary.

For further information refer to the documentation files parport.txt,

modules.txt, kmod.txt and README provided with recent kernel sources,

or ask a real Linux expert!

(See also chapter "III.4 Shell Script Operation under Linux".)

6. The boot script aborts with the error message

resetting target system ...

./upload: /dev/ttyS4: Device or resource busy

The interrupt of the serial interface may already be used by another

driver (i.e. printer port, USB).

Try to find and solve the interrupt conflict. (See items 2 and 5.)

Appendix A:     CUSTOMIZ Error Messages

===========

A.1 Parameter Errors:

---------------------

Parameter errors apply to the correctness and consistency of the command

line parameters. If one of these errors is detected, it is flagged on the

console, and CUSTOMIZ is aborting with exit code 1:

accuracy out of range                 Baudrate accuracy < 0, or > 5 %.

address out of range                  Address is no unsigned 16-bit number.

baudrate out of range                 Specified baudrate is < 0.

clock frequency out of range          Specified clock frequency is < 0.

illegal real number                   Parameter is no valid real number.

invalid hex number                    Parameter is no valid hex number.

too many parameters                   More than 6 parameters specified.

unknown baudrate generator            Baudrate generator not implemented.

A.2 Baudrate Errors:

--------------------

Baudrate errors apply to the results that CUSTOMIZ has derived from the

given program parameters. If one of these errors is detected, it is flagged

on the console, and CUSTOMIZ is aborting with exit code 1:

baudrate cannot be generated          Baudrate error is greater than 10 %.

insufficient accuracy                 Baudrate error is greater than the

required accuracy.

A.3 Runtime Errors:

-------------------

In general, runtime errors are file I/O errors.

If one of these errors is detected, it is flagged on the console,

and CUSTOMIZ is aborting with exit code 2:

access denied                 No privilege for attempted operation.

disk full                     No more free disk space.

disk write-protected          Attempt to write to a write-protected disk.

drive not ready               Disk drive is off, or no media mounted.

fatal I/O error               General (unknown) disk or device I/O error.

Appendix B:     RESET51 Error Messages

===========

General Errors:

too many parameters           More than two parameters specified.

illegal port name             Unknown device name.

invalid pulse-width           Pulse width is 0, or no unsigned 16-bit number.

hardware failure              The port register cannot be read back.

MS-DOS only:

port not found                The BIOS did not recognize the port.

Linux only:

no root privilege             Program doesn't run as root.

access denied                 No r/w access to character device.

port not found                Port entry not found in /proc/parport

invalid base address          Format error in /proc/parport/n/hardware

Appendix C:     BOOT-51 Error Messages

===========

checksum error                Intel-HEX record checksum is not correct.

illegal command               A command other than U or G has been entered.

illegal hex digit             Hex digit expected. Another character received.

illegal record ID             Intel-HEX record type is greater than 1.

unexpected character          The received Intel-HEX record doesn't start

with ':', or doesn't end with CR, LF, or CR/LF.

Appendix D:     Trademarks

===========

ASEM-51 is a trademark of W.W. Heinz.

MCS-51 is a trademark of Intel Corporation.

MCB-517 is a trademark of Keil Elektronik GmbH.

TELIX is a trademark of deltaComm Development.

IBM-PC, IBM-XT, and IBM-AT are trademarks of IBM Corporation.

MS-DOS is a trademark of Microsoft Corporation.

Windows is a trademark of Microsoft Corporation.

Linux is a trademark of Linus Torvalds.

All device codes of 8051 derivatives are trademarks of the manufacturers.

Other brand and product names are trademarks of their respective holders.

Appendix E:     Tables of Baudrates

===========

The following tables show the most frequently used baudrates that can

be generated with a particular timer or baudrate generator, for some

standard clock frequencies.

Baudrates which can be generated with a relative accuracy of better

than 4.0 %, are marked with that accuracy (in %) or with a '+' character,

if they can be generated exactly (better than 0.01 %).

Baudrates that can be generated with Timer 1:

---------------------------------------------

bps\MHz | 7.3728  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+----------------------------------------------------------------------

128000 |                                                                 2.34

76800 |                              +

64000 |                    2.34                                         2.34

57600 |            +

38400 |   +                          +

31250 |                     +                          +                 +

19200 |   +        +                 +                2.34

14400 |            +                         3.55              3.34     3.55

12500 |  2.40               +       2.40                                 +

9600 |   +        +                 +       3.55     2.34     1.36     0.16

7200 |            +       3.55     3.03     3.55     0.16     3.34     2.12

4800 |   +        +       0.16      +       2.12     2.34     1.36     0.16

3600 |  3.03      +       2.12     1.59     0.64     0.16     0.22     0.79

2400 |   +        +       0.16      +       0.79     0.16     0.94     0.16

1800 |  1.59      +       0.79     0.78     0.64     0.16     0.22     0.64

1200 |   +        +       0.16      +       0.64     0.16     0.22     0.16

600 |   +        +       0.16      +       0.08     0.16     0.22     0.16

300 |   +        +       0.16      +       0.08     0.16     0.22     0.16

200 |   +        +       0.16      +       0.16     0.16     1.73

150 |   +        +       0.16      +

134 |  0.20     0.03     0.09

110 |  0.26     2.27

75 |   +

Baudrates that can be generated with Timer 2 of the 8052:

---------------------------------------------------------

bps\MHz | 7.3728  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+----------------------------------------------------------------------

460800 |                              +

256000 |                                      2.34                       2.34

230400 |   +                          +

128000 |                    2.34              2.34              2.34     2.34

115200 |   +        +                 +                2.34

76800 |   +                2.34      +                         1.73     2.34

64000 |                    2.34     2.86     2.34     2.34     2.34     2.34

57600 |   +        +                 +       3.55     2.34     1.36     0.16

38400 |   +        +       2.34      +       0.16     2.34     1.73     2.34

31250 |           0.54      +       1.70      +        +        +        +

19200 |   +        +       2.34      +       0.16     1.02     1.36     0.16

14400 |   +        +       0.16      +       0.79     0.16     0.94     0.16

12500 |  2.40     1.26      +       0.37      +        +        +        +

9600 |   +        +       0.16      +       0.16     0.69     0.16     0.16

7200 |   +        +       0.16      +       0.64     0.16     0.22     0.16

4800 |   +        +       0.16      +       0.16     0.16     0.16     0.16

3600 |   +        +       0.16      +       0.08     0.16     0.22     0.16

2400 |   +        +       0.16      +       0.16     0.16     0.16     0.16

1800 |   +        +       0.16      +       0.08     0.16     0.06     0.08

1200 |   +        +       0.16      +       0.08     0.05     0.03      +

600 |   +        +        +        +       0.04     0.05     0.03      +

300 |   +        +        +        +       0.02      +       0.02      +

200 |   +        +        +        +        +       0.02      +        +

150 |   +        +        +        +       0.01      +        +        +

134 |  0.02      +       0.02      +        +        +        +        +

110 |  0.02      +        +        +       0.01      +        +        +

75 |   +        +        +        +        +        +        +        +

50 |   +        +        +        +        +        +        +        +

Baudrates that can be generated with the 80535 Internal Baudrate Generator:

---------------------------------------------------------------------------

bps\MHz | 7.3728  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+----------------------------------------------------------------------

19200 |                                                                  +

14400 |                                                +

12500 |                                      2.40

9600 |                     +                                            +

7200 |                                                +

4800 |                     +

Baudrates that can be generated with the 80C515A Internal Baudrate Generator:

-----------------------------------------------------------------------------

bps\MHz | 7.3728  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+----------------------------------------------------------------------

460800 |                              +

256000 |                                      2.34                       2.34

230400 |   +                          +

128000 |                    2.34              2.34              2.34     2.34

115200 |   +        +                 +                2.34

76800 |   +                2.34      +                         1.73     2.34

64000 |                    2.34     2.86     2.34     2.34     2.34     2.34

57600 |   +        +                 +       3.55     2.34     1.36     0.16

38400 |   +        +       2.34      +       0.16     2.34     1.73     2.34

31250 |           0.54      +       1.70      +        +        +        +

19200 |   +        +       2.34      +       0.16     1.02     1.36     0.16

14400 |   +        +       0.16      +       0.79     0.16     0.94     0.16

12500 |  2.40     1.26      +       0.37      +        +        +        +

9600 |   +        +       0.16      +       0.16     0.69     0.16     0.16

7200 |   +        +       0.16      +       0.64     0.16     0.22     0.16

4800 |   +        +       0.16      +       0.16     0.16     0.16     0.16

3600 |   +        +       0.16      +       0.08     0.16     0.22     0.16

2400 |   +        +       0.16      +       0.16     0.16     0.16     0.16

1800 |   +        +       0.16      +       0.08     0.16     0.06     0.08

1200 |   +        +       0.16      +       0.08     0.05     0.03      +

600 |   +        +        +        +       0.04     0.05     0.03      +

300 |   +        +        +        +       0.04     0.05     1.73

200 |   +        +       0.05

150 |   +

134 |  0.03

110 |  2.27

Baudrates that can be generated with Timer 1 of the DALLAS 80C320:

------------------------------------------------------------------

bps\MHz | 7.3728  11.0592  12.0000  14.7456  16.0000  18.0000  20.0000  24.0000

--------+----------------------------------------------------------------------

256000 |                                      2.34

230400 |                              +

128000 |                                      2.34                       2.34

115200 |   +                          +

76800 |                              +                         1.73     2.34

64000 |                    2.34              2.34              2.34     2.34

57600 |   +        +                 +                2.34

38400 |   +                2.34      +                         1.73     2.34

31250 |                     +                 +        +        +        +

19200 |   +        +       2.34      +       0.16     2.34     1.73     2.34

14400 |   +        +       0.16      +       2.12     2.34     1.36     0.16

12500 |  2.40     1.26      +       2.40      +       2.17      +        +

9600 |   +        +       2.34      +       0.16     1.02     1.36     0.16

7200 |   +        +       0.16      +       0.79     0.16     0.94     0.16

4800 |   +        +       0.16      +       0.16     0.69     0.16     0.16

3600 |   +        +       0.16      +       0.64     0.16     0.22     0.16

2400 |   +        +       0.16      +       0.16     0.16     0.16     0.16

1800 |   +        +       0.16      +       0.08     0.16     0.22     0.16

1200 |   +        +       0.16      +       0.16     0.16     0.16     0.16

600 |   +        +       0.16      +       0.08     0.16     0.22     0.16

300 |   +        +       0.16      +       0.08     0.16     0.22     0.16

200 |   +        +       0.16      +       0.16     0.16     1.73

150 |   +        +       0.16      +

134 |  0.20     0.03     0.09

110 |  0.26     2.27

75 |   +