lcdpanel - device driver for running a Hitachi HD44780 LCD panel from a PC

parallel port

version .50 - Not great code, but it now compiles on new kernels..

(and I fixed some lame stuff I did)

------

Copyright 1991-1995 Stephen S. Richardson (prefect@wpi.edu)

based off of several versions for several hardware and software platforms

from 1991-1995.

(credit is due to Arthur C. Smith, whose joystick code I studied to learn how

to write linux device drivers, and to Linus Torvalds, for being the cool kind

of person to start the whole Linux project.. yar.)

You may modify, copy, fix, change, or delete this code, as long as my

copyright remains intact.  Well, if you delete it, it doesn't have to.  I also

ask that you send me a piece of email if you make any interesting changes.

You don't have to, but I want to see if anyone does anything cool with it.

There are no warranties, expressed or implied.  If your computer blows up, or

you blow up your sibling trying to use this, I'm not responsible.  If it works,

and all your friends thing you're cool, you must tell them it is due to this

project. ;)

--------------------------------

Hardware:

You'll need a free parallel port, and an LCD module.  This code is written

to work with a Hitachi LM108L (40x2 backlit) LCD, but with minor modifications

will work with other Hitachi-chipset LCD panels.

A source for the panels is:

TIMELINE, INC.

23605 TELO AVENUE

TORRANCE, CA 90505

(310) 784-5488

(310) 784-7590 (fax)

Timeline has several models available.  Call for information, I'm not going

to reprint their catalog here..

You'll also need some miscellaneous stuff like 14-conductor ribbon cable,

a 50K linear taper potentiometer, a source for +5v and -5v (you can pull it

off of the computer's power supply if you're into that sort of thing).  Some

LCD panels come with a row of header pins on them.. The most elegant way to

connect to this is with a 14-pin IDC connector.  You'll also need a male 25

pin DB25 connector, and access to a soldering iron and solder.

Here are the pinouts of the standard Hitachi LCD panels.. (consult the data

sheet you get with yours to verify)

Pin  Symbol   Function

1    Vss       0v (gnd)

2    Vdd       +5

3    Vd        contrast voltage

4    RS        H: data input   L: instruction code input

5    R/W       H: data read (LCD->CPU)  L: data write (CPU->LCD)

6    E         H, H->L enable

7-14 DB0-DB7   Data bits 0-7 (0=lsb)

Here are the pinouts of a standard PC parallel port (the important ones)

Pin  Symbol   Function

1    Strobe    connected to the Enable signal of the LCD

2-9  DB0-DB7   connect to DB0-DB7 on LCD

14   Afeed     connected to the RS input of the LCD

25   gnd       connected to gnd on the LCD

Connect pin 5 of the LCD (R/W) to gnd.  I don't bother reading back the data

in the LCD's 80 byte RAM.  Besides, most people's parallel ports aren't

bidirectional.

You can get +5 from your power supply connector inside your computer.  It's

*usually* the red wires coming out of the supply.  I do NOT guarantee this.

Measure it.  The same goes for -5.  It's *usually* white.  I can't guarantee

anything.  Measure measure measure.  Or, if you're feeling less-than-

adventurous, get a +5/-5 supply from somewhere and connect its gnd to the PC

gnd, and use that.  Connect +5 to the +5 pin on the LCD.

Now, here comes the tricky part.  You may not need -5v.  A long time ago, I

ran a 16x1 LCD and got decent contrast without -5v, but my 40x2 needs it.

I know this probably makes little sense to you, but I don't want to get in

to technical details right now.  Check your data sheets, and experiment.

Anyway, this is what I use for my 40x2:

+5 -------------------===( 50K linear pot)

(               )

to contrast in on LCD ----===(  (wiper)      )

(               )

-5 -------------------===(               )

Yes, it's an awful picture.  Deal.  With a 16x1, i was able to substitute

gnd for the -5.

Anyway.  Hook all this business up.  If you're confused at my complete lack

of coherency, or just don't grok the ee speak, talk to someone who does, or

wait until I have time to make a better doc file on this.

As I was saying.  Hook it up.  If you've got the exact same setup as I do,

it should work.  Most likely it'll take some kludging and work to get it to

run.. Sorry for this, but I really wrote the whole thing quickly and have

not had time to fix it.

Software:

Well, I guess you could call it that anyway.  lcd.c is the device driver

code.  lcdd.c is a daemon that I wrote (it doesn't fork off and turn into

a background process yet..) that displays some interesting stuff.  Actually,

now that I think of it, there's more stuff in the code than actually gets

displayed when it runs.. lcd.c is kinda crufty, and lcdd.c is very crufty.

It was a quick hack, and one of these days I'll fix it.

use insmod to load lcd.o once it's compiled.

Oh yeah.. remoted.c is a program that ran on one of my other boxes, and

dumped stuff to a cross-mounted directory.. lcdd picks it up and displays

status of the two machines..  It's kludgy.

Please don't bomb me with email asking for billions of features.  I'm not

likely to have time to work on this until late spring or summer.  I'm just

releasing this so someone can build off of my work and get something better

written.

Well, I hope that's everything.. I know it seems like a lot of stuff.. But

hopefully it will make sense to someone.

If you want to see my old way-hacked Linux boxes, fire up a Web browser and

check out:

"http://sidehack.gweep.net/~prefect/monstertower.html"

Welp, I guess that's it.

Hack on.

Steve

prefect@sidehack.gweep.net

prefect@wpi.edu

(fnord)